How to Download Arduino, Drivers and Libraries

Content

1. Install Windows Drivers	1

1.1 Download and install Arduino software	2

1.2 Install the drivers on the Windows system	7

1.3 Arduino IDE setting	13

1.4 Start first program	17

2. Install the Drivers on the MAC System	23

2.1 Download and install Arduino software	23

2.2 Download the drivers of CP2102	24

2.3 How to install the drivers of CP2102	24

3. Install Library Files to Arduino	37

3.1 Manual installation	38

# 1. Install Windows Drivers

1.1 Download and install Arduino software

Enter the Arduino official website: https://www.arduino.cc/ and click "SOFTWARE" to enter the download page, as shown in the following figure.

Then, select and download the corresponding installer according to your operating system. If you are a Windows user, please select "Windows Installer" to download the correct installation driver.

Click Windows Win7 and newer to download the installer for Arduino 1.8.16, which needs to be installed manually. And when clicking on the Windows ZIP file, the zip file of the Arduino 1.8.16 will be downloaded directly, and you only need to unzip it to complete the installation.

Generally, you can download it by clicking JUST DOWNLOAD. Of course, if you like, you can choose a small sponsorship to help the great Arduino open source.

(3) Once the Arduino software is downloaded, continue to install. When you receive a warning from your operating system, allow the driver to install. Click on I Agree, select the components you want to install and then click Next.

- Select the installation directory (The default directory is recommended.), then click Install.

- If the following interface appears, you should select Install.

This process will extract and install all the necessary files to properly execute the Arduino software (IDE).

After the installation is complete, an Arduino software shortcut will be created on the desktop.

1.2.Install the Pico development board

Open Arduino IDE，and click Tools→Board→Boards Manager...

Enter Pico and select Arduino Mbed OS RP2040 Boards and click Install

- Then click Install

Then click Close

1.3 Upload the pico firmware compatible with Arduino

You need to upload a pico firmware compatible with Arduino.

Disconnect the computer with the pico board.

Hold down the button BOOTSEL, connect them again before releasing the button.

Keep holding down the button before connecting the pico board with your computer; otherwise, the firmware can’t be downloaded

（2）Open Arduino IDE and File→Examples→01.Basics→Blink

Click Tools→Board→Arduino Mbed OS RP2040 Boards→Raspberry Pi Pico

Upload the script（Blink）to the Raspberry Pi Pico

Then the indicator on the Raspberry Pi Pico will flash.

1S

Click Tools→Port→COMx(Raspberry Pi Pico)

Com port is different on the computers.

Please select your correct COM port. In this tutorial, the com port is Com15.

Don’t select the port if it’s your first time to upload scripts to the pico board .

Select if the com port is correct when uploading the script each time; otherwise the code will fail to upload.

Sometimes, Raspberry Pi Pico can’t work in light of the loss of code. At this time, you need to upload the firmware of the pico board in accordance with above steps

- Install the driver on MAC

Download Arduino IDE:

Just refer to the Windows system