Project 13：Passive Buzzer

- Introduction

In a previous project, we have learned an active buzzer, which can only produce one sound and may let you feel monotonous. In this project, we will learn a passive buzzer and use the Raspberry Pi Pico to control the passive buzzer to sound an alarm. Unlike the active buzzer, the passive buzzer can emit sounds of different frequencies.

- Components Required

| Raspberry Pi Pico*1 |  | Raspberry Pi PicoExpansion Board*1 |  |
| --- | --- | --- | --- |
| Passive Buzzer*1 | Breadboard*1 | Jumper Wires | USB Cable*1 |

3.Component Knowledge

A passive buzzer is an integrated electronic buzzer with no internal vibration source. It must be driven by 2K to 5K square wave, not a DC signal. The two buzzers are very similar in appearance, but one buzzer with a green circuit board is a passive buzzer, while the other with black tape is an active buzzer. Passive buzzers cannot distinguish between positive polarity while active buzzers can.

- 4. Circuit Diagram and Wiring Diagram

- Test Code

- The code used in this project is saved in the file KS3020 Keyestudio Raspberry Pi Pico Learning Kit Ultimate Edition\3. Raspberry Pi System\Python_Tutorial\2. Projects\Project 13：Passive Buzzer. You can move the code anywhere. We save the code to the pi folder of the Raspberry Pi system. The path:home/pi/2. Projects

Open“Thonny”, click“This computer”→“home”→“pi”→“2. Projects”→“Project 13：Passive Buzzer”. And double left-click the“Project_13_Passive_Buzzer.py”.

from machine import Pin import time #Initialize the passive buzzer buzzer = Pin(16,Pin.OUT) #Simulate two different frequencies while True: #Output 500HZ frequency sound for i in range(80): buzzer.value(1) time.sleep(0.001) buzzer.value(0) time.sleep(0.001) #Output 250HZ frequency sound for i in range(100): buzzer.value(1) time.sleep(0.002) buzzer.value(0) time.sleep(0.002)

- Test Result

- Ensure that the Raspberry Pi Pico is connected to the computer，click“Stop/Restart backend”.

- Click “Run current script”, the code starts executing, we will see that the the passive buzzer sounds the alarm. Click “Stop/Restart backend”to exit the program.