Project 18：Small Fan

- Introduction

In the hot summer, we need an electric fan to cool us down, so in this project, we will use a Raspberry Pi Pico to control 130 motor module and small blade to make a small fan.

- Components Required

| Raspberry Pi Pico*1 | Raspberry Pi Pico Expansion Board*1 |  |
| --- | --- | --- |
| 130 Motor Module*1 | M-F Dupont Wires | USB Cable*1 |

- Component Knowledge

130 motor module: The motor control module uses the HR1124S motor control chip, which is a single-channel H-bridge driver chip for DC motor. The H-bridge driving part of the HR1124S features low on-resistance PMOS and NMOS power tube. The low on-resistance ensures low power loss of the chip, making the chip work safely for a longer time. In addition, HR1124S has low standby current and low quiescent current, which makes HR1124S easy to be used in toy scheme.

Features:

Working voltage: 5V

Working current: 200MA

Working power: 2W

Working temperature: -10℃~ +50℃

Schematic diagram:

- Circuit Diagram and Wiring Diagram

- Test Code

The code used in this project is saved in the file S3020 Keyestudio Raspberry Pi Pico Learning Kit Ultimate Edition\3. Raspberry Pi System\Python_Tutorial\2. Projects\Project 18：Small Fan.

You can move the code anywhere. We save the code to the pi folder of the Raspberry Pi system. The path:home/pi/2. Projects

Open“Thonny”, click““This computer”→“home”→“pi”→“2. Projects”→“Project 18: Small Fan”. And double-click the“Project_18_ Small_Fan.py”.

from machine import Pin import time motor1a = Pin(17, Pin.OUT) # create motor1a object from Pin 17, Set Pin 17 to output motor1b = Pin(16, Pin.OUT) # create motor1b object from Pin 16, Set Pin 16 to output def forward(): motor1a.high() # Set motor1a high motor1b.low() # Set motor1b low def backward(): motor1a.low() motor1b.high() def stop(): motor1a.low() motor1b.low() def test(): forward() # motor forward time.sleep(5) #delay stop() # motor stop time.sleep(2) backward()# motor backward time.sleep(5) stop() time.sleep(2) for i in range(5): test()

- Test Result

- Ensure that the Raspberry Pi Pico is connected to the computer，click“Stop/Restart backend”.

- Click “Run current script”, the code starts executing, we will see that The small fan turns counterclockwise for 5 seconds and stops for 2 seconds, and then turns clockwise for 5 seconds and stops for 2 seconds. Repeat this rule for 5 times and then the small fan stops. Click“Stop/Restart backend”to exit the program.